Reconstruction of a local road from the intersection with the road to dam Turija to the center of the village of Dobrosinci-Municipality of Vasilevo

Total road length: L = 1320 m

- Design speed: 30 km/h

- Road profile width: 5.00 m

- Maximum cross slope: 2.50 %

- Maximum longitudinal slope: 10.69 %

Pavement Structure

- BNHS 16 – asphalt layer: d = 7cm

- Base layer of buffer layer under the roadway: 35cm

- Base layer of buffer layer under the sidewalks: 20cm

The subject street is part of Т4D5.

The work is being carried out by the construction  DPTU ЕSKAVATORI – MK Dooel Skopje. The value of the signed contract is 16,421,768 denars.

# Project Overview

Preparatory work:

- Securing and marking the route

Lower and upper structure:

- Mechanical excavation of humus, with transport to the landfill (for road widening)

- Mechanical excavation of the existing roadway

- Subsoil/bed preparation

- Embankment preparation

- Installation of a buffer layer (under the roadway) d=35cm

- Installation of concrete curbs 18/24

- Installation of buffer layer (under sidewalks) d=20cm

- Installation of BNHS layer 16 d=8cm

- Installation of sidewalk concrete curbs 8/22

- Installation of paver elements on sidewalks d=6cm

Drainage:

- Installation of drainage pipes f150 (with excavation, filter material and backfilling

- Drainage from km.0+902-1+32

Traffic signals:

- Installation of traffic signs

- Marking of horizontal signaling

Additional quantities:

- Construction of an embankment, with procurement of the material (improved subgrade)

- Construction of two drainage outlets at km.0+958 and km.1+026 in accordance with the Technical Solution for Road Drainage.