Construction of local road,, Gavrilo Gavrilski”- Municipality of Sveti Nikole

Total road length: L = 332.84 m

- Design speed: 30 km/h

- Road profile width: 5.50 m

- Maximum cross slope: 2.50 %

- Maximum longitudinal slope: 9.35%

Pavement Structure

- BNHS 22 – asphalt layer: d = 7cm

- Base layer of buffer layer under the roadway: 30cm

- Base layer of buffer layer under the sidewalks: 20cm

The subject street is part of Т4D5.

The work is being carried out by the construction  DPTU ЕSKAVATORI – MK Dooel Skopje. The value of the signed contract is 7,840,222 denars.

# Project Overview

Preparatory work:

- Securing and marking the route

- Dislocation of existing concrete columns

Lower and upper structure:

- Mechanical excavation of earth in a wide excavation 3-4 floors.

- Making a bed

- Making an embankment

- Leveling existing manholes

- Installing a buffer layer (under the roadway)d=30см

- Installation of concrete curbs18/24

- Installation of buffer layer (under sidewalks) d=20см.

- Installation of BNS22 layer d=7см.

- Installation of AB11 layer d=5см.

- Installation of sidewalk concrete curbs 6/20

- Installation of paver elements on sidewalks

Drainage:

- Installation of drainage pipes f150 (with excavation, filter material and backfilling

- Procurement and installation of street drains

- Production of absorption wells (4 pcs.)

Traffic signals:

- Installation of traffic signs

- Marking of horizontal signaling

Additional quantities:

- Construction of an embankment, with procurement of the material (improved bedding)

- Procurement and installation of iron manhole covers

| Street “Gavrilo Gavrilski” | Street “Gavrilo Gavrilski” |
| --- | --- |
| Street “Gavrilo Gavrilski” | Street “Gavrilo Gavrilski” |
| Street “Gavrilo Gavrilski” |  |